## Integrated Development Environments (IDEs) Used

| Component |  | IDE Used |
| --- | --- | --- |
| Arduino Nano Controller |  | Arduino IDE |
| Arduino Uno Robot Cars |  | Arduino IDE |
| ESP32 Arena Controller |  | Arduino IDE |
| ESP32 Web Interface |  | VSCode (HTML, CSS, JavaScript embedded in ESP32 code) |

## Programming Languages Used

| Component | Language |
| --- | --- |
| Arduino Nano Controller | C/C++ (Arduino Framework) |
| Arduino Uno Robot Cars | C/C++ (Arduino Framework) |
| ESP32 Arena Controller | C/C++ (Arduino Framework) |
| Arena Web Interface | HTML, CSS, JavaScript |

Car Code

#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

#include <Servo.h>

RF24 radio(9, 10); // CE, CSN

const byte address[6] = "00001"; // change to 00002 for Car 2

Servo myServo;

// Data from transmitter

struct DataPacket {

int x;

int y;

bool button;

int x2;         // ignored by car

bool goalieBtn; // ignored by car

};

DataPacket data;

// Motor driver pins (L298N)

const int ENA = 5;

const int IN1 = 2;

const int IN2 = 4;

const int IN3 = 7;

const int IN4 = 8;

const int ENB = 6;

// Servo

const int servoPin = 3;

bool prevButton = false;

void setup() {

Serial.begin(9600);

radio.begin();

radio.openReadingPipe(0, address);

radio.setPALevel(RF24_PA_LOW);

radio.startListening();

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

myServo.attach(servoPin);

myServo.write(0);

Serial.println("Car Receiver Ready");

}

void loop() {

if (radio.available()) {

radio.read(&data, sizeof(data));

handleMovement();

handleServo();

}

}

void handleMovement() {

int xValue = data.x;

int yValue = data.y;

int xMapped = map(xValue, 0, 1023, -512, 512);

int yMapped = map(yValue, 0, 1023, 512, -512); // inverted to fix forward/backward

int deadZone = 100;

if (abs(xMapped) < deadZone) xMapped = 0;

if (abs(yMapped) < deadZone) yMapped = 0;

int leftSpeed = yMapped + xMapped;

int rightSpeed = yMapped - xMapped;

leftSpeed = constrain(leftSpeed, -512, 512);

rightSpeed = constrain(rightSpeed, -512, 512);

setMotor(ENA, IN1, IN2, leftSpeed);

setMotor(ENB, IN3, IN4, rightSpeed);

}

void setMotor(int EN, int IN1, int IN2, int speedVal) {

if (speedVal > 0) {

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

} else if (speedVal < 0) {

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

} else {

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

}

analogWrite(EN, map(abs(speedVal), 0, 512, 0, 255));

}

void handleServo() {

if (data.button && !prevButton) {

myServo.write(90);

delay(250);

myServo.write(0);

}

prevButton = data.button;

}

Controller Code

#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

RF24 radio(9, 10); // CE, CSN

const byte address[6] = "00001";

// Pins

#define JOY_X A0

#define JOY_Y A1

#define KICKER_BTN 4

#define GOALIE_X A2

#define GOALIE_BTN 5

struct DataPacket {

int x;

int y;

bool button;

int x2;

bool goalieBtn;

};

DataPacket data;

void setup() {

pinMode(KICKER_BTN, INPUT_PULLUP);

pinMode(GOALIE_BTN, INPUT_PULLUP);

radio.begin();

radio.openWritingPipe(address);

radio.setPALevel(RF24_PA_LOW);

radio.stopListening();

}

void loop() {

data.x = analogRead(JOY_X);

data.y = analogRead(JOY_Y);

data.button = (digitalRead(KICKER_BTN) == LOW);

data.x2 = analogRead(GOALIE_X);

data.goalieBtn = (digitalRead(GOALIE_BTN) == LOW);

radio.write(&data, sizeof(data));

delay(20);

}

Arena Code

/*

* Football Arena Controller — FINAL VERSION

* Hardware: ESP32 + 2x IR Break Beam Sensors + 2x LED Strips (90 bulbs each) + Active Piezo Buzzer

*

* PIN LAYOUT:

*   IR Sensor 1 (Player 1 goal)  -> GPIO 26

*   IR Sensor 2 (Player 2 goal)  -> GPIO 27

*   LED Strip 1 (Player 1 side)  -> GPIO 5

*   LED Strip 2 (Player 2 side)  -> GPIO 18

*   Piezo Buzzer                 -> GPIO 19

*

* WIRING:

*   IR Sensors : VCC -> 3.3V, GND -> GND, OUT -> GPIO

*   LED Strips : External 5V PSU -> Strip VCC

*                External GND + ESP32 GND joined together -> Strip GND

*                470Ω resistor on data line (GPIO -> Strip DIN)

*   Buzzer     : GPIO 19 -> positive leg, GND -> negative leg

*                HIGH = ON (active buzzer)

*/

#include <WiFi.h>

#include <WebServer.h>

#include <FastLED.h>

#include <ArduinoJson.h>

// ---- WiFi Credentials ----

const char* ssid     = "REDMI 15C";

const char* password = "123456789";

// ---- Pin Definitions ----

#define IR_SENSOR_1   26

#define IR_SENSOR_2   27

#define LED_STRIP_1    5

#define LED_STRIP_2   18

#define BUZZER_PIN    19

// ---- LED Config ----

#define NUM_LEDS        90

#define LED_TYPE        WS2812B

#define COLOR_ORDER     GRB

#define LED_BRIGHTNESS  80        // Keep low until external PSU is wired properly

CRGB strip1[NUM_LEDS];

CRGB strip2[NUM_LEDS];

// ---- Web Server ----

WebServer server(80);

// ---- Game State ----

struct GameState {

bool          active     = false;

bool          paused     = false;

int           timeLeft   = 0;

int           score1     = 0;

int           score2     = 0;

String        name1      = "Player 1";

String        name2      = "Player 2";

int           totalTime  = 0;

unsigned long lastTick   = 0;

unsigned long pauseEnd   = 0;

int           goalWinner = 0;

};

GameState game;

// ---- IR Edge Detection ----

bool sensor1Clear = true;

bool sensor2Clear = true;

// ---- LED task state ----

// We drive LEDs from a separate FreeRTOS task to avoid WiFi interference

enum LedMode {

LED_OFF,

LED_RAINBOW,

LED_WHITE,

LED_GOAL_1,    // strip1 red, strip2 off

LED_GOAL_2,    // strip2 red, strip1 off

LED_WIN_1,     // strip1 green, strip2 red

LED_WIN_2,     // strip1 red, strip2 green

LED_DRAW       // both yellow

};

volatile LedMode ledMode   = LED_RAINBOW;  // rainbow on power-on

volatile uint8_t rainbowHue = 0;

// ---- Buzzer (non-blocking) ----

bool          buzzerActive = false;

unsigned long buzzerEnd    = 0;

// HIGH = ON for this hardware

void buzzerOn()  { digitalWrite(BUZZER_PIN, HIGH); }

void buzzerOff() { digitalWrite(BUZZER_PIN, LOW);  }

void startBuzzer(unsigned long ms) {

buzzerOn();

buzzerActive = true;

buzzerEnd    = millis() + ms;

}

void updateBuzzer() {

if (buzzerActive && millis() >= buzzerEnd) {

buzzerOff();

buzzerActive = false;

}

}

// =========================================================

//  LED Task — runs on Core 0, WiFi runs on Core 1

//  This prevents FastLED timing from being disrupted by WiFi

// =========================================================

void ledTask(void* param) {

FastLED.addLeds<LED_TYPE, LED_STRIP_1, COLOR_ORDER>(strip1, NUM_LEDS).setCorrection(TypicalLEDStrip);

FastLED.addLeds<LED_TYPE, LED_STRIP_2, COLOR_ORDER>(strip2, NUM_LEDS).setCorrection(TypicalLEDStrip);

FastLED.setBrightness(LED_BRIGHTNESS);

fill_solid(strip1, NUM_LEDS, CRGB::Black);

fill_solid(strip2, NUM_LEDS, CRGB::Black);

FastLED.show();

for (;;) {

switch (ledMode) {

case LED_RAINBOW:

for (int i = 0; i < NUM_LEDS; i++) {

strip1[i] = CHSV(rainbowHue + (i * 256 / NUM_LEDS), 255, 200);

strip2[i] = CHSV(rainbowHue + (i * 256 / NUM_LEDS), 255, 200);

}

FastLED.show();

rainbowHue++;

vTaskDelay(20 / portTICK_PERIOD_MS);

break;

case LED_WHITE:

fill_solid(strip1, NUM_LEDS, CRGB::White);

fill_solid(strip2, NUM_LEDS, CRGB::White);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_GOAL_1:

fill_solid(strip1, NUM_LEDS, CRGB::Red);

fill_solid(strip2, NUM_LEDS, CRGB::Black);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_GOAL_2:

fill_solid(strip2, NUM_LEDS, CRGB::Red);

fill_solid(strip1, NUM_LEDS, CRGB::Black);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_WIN_1:

fill_solid(strip1, NUM_LEDS, CRGB::Green);

fill_solid(strip2, NUM_LEDS, CRGB::Red);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_WIN_2:

fill_solid(strip1, NUM_LEDS, CRGB::Red);

fill_solid(strip2, NUM_LEDS, CRGB::Green);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_DRAW:

fill_solid(strip1, NUM_LEDS, CRGB::Yellow);

fill_solid(strip2, NUM_LEDS, CRGB::Yellow);

FastLED.show();

vTaskDelay(50 / portTICK_PERIOD_MS);

break;

case LED_OFF:

default:

fill_solid(strip1, NUM_LEDS, CRGB::Black);

fill_solid(strip2, NUM_LEDS, CRGB::Black);

FastLED.show();

vTaskDelay(100 / portTICK_PERIOD_MS);

break;

}

}

}

// =========================================================

//  CORS

// =========================================================

void addCORS() {

server.sendHeader("Access-Control-Allow-Origin",  "*");

server.sendHeader("Access-Control-Allow-Methods", "GET, POST, OPTIONS");

server.sendHeader("Access-Control-Allow-Headers", "Content-Type");

}

// =========================================================

//  API: POST /api/start

// =========================================================

void handleStart() {

addCORS();

if (server.method() == HTTP_OPTIONS) { server.send(204); return; }

if (!server.hasArg("plain")) {

server.send(400, "application/json", "{\"error\":\"no body\"}");

return;

}

StaticJsonDocument<256> doc;

if (deserializeJson(doc, server.arg("plain"))) {

server.send(400, "application/json", "{\"error\":\"bad json\"}");

return;

}

game.name1      = doc["name1"] | "Player 1";

game.name2      = doc["name2"] | "Player 2";

game.totalTime  = doc["time"]  | 60;

game.timeLeft   = game.totalTime;

game.score1     = 0;

game.score2     = 0;

game.active     = true;

game.paused     = false;

game.lastTick   = millis();

game.goalWinner = 0;

sensor1Clear = true;

sensor2Clear = true;

// Both strips white when game starts

ledMode = LED_WHITE;

// Beep to signal game start

startBuzzer(500);

server.send(200, "application/json", "{\"status\":\"started\"}");

Serial.println("Game started!");

}

// =========================================================

//  API: POST /api/stop

// =========================================================

void handleStop() {

addCORS();

game.active  = false;

game.paused  = false;

buzzerOff();

buzzerActive = false;

ledMode      = LED_RAINBOW;   // back to rainbow when stopped

server.send(200, "application/json", "{\"status\":\"stopped\"}");

}

// =========================================================

//  API: GET /api/state

// =========================================================

void handleState() {

addCORS();

StaticJsonDocument<256> doc;

doc["active"]   = game.active;

doc["paused"]   = game.paused;

doc["timeLeft"] = game.timeLeft;

doc["score1"]   = game.score1;

doc["score2"]   = game.score2;

doc["name1"]    = game.name1;

doc["name2"]    = game.name2;

doc["total"]    = game.totalTime;

doc["winner"]   = game.goalWinner;

String out;

serializeJson(doc, out);

server.send(200, "application/json", out);

}

void handleOptions() {

addCORS();

server.send(204);

}

// =========================================================

//  Goal trigger

// =========================================================

void triggerGoal(int player) {

if (game.paused || !game.active) return;

if (player == 1) {

game.score1++;

sensor1Clear = false;

ledMode      = LED_GOAL_1;   // strip1 red, strip2 off

} else {

game.score2++;

sensor2Clear = false;

ledMode      = LED_GOAL_2;   // strip2 red, strip1 off

}

startBuzzer(3000);             // buzzer on for 3 seconds

game.paused     = true;

game.goalWinner = player;

game.pauseEnd   = millis() + 3000;

Serial.printf("GOAL! Player %d  |  Score: %d - %d\n", player, game.score1, game.score2);

}

// =========================================================

//  Resume after 3s pause

// =========================================================

void resumeGame() {

game.paused     = false;

game.goalWinner = 0;

sensor1Clear    = true;

sensor2Clear    = true;

ledMode         = LED_WHITE;   // both strips white

startBuzzer(200);              // short beep on resume

Serial.println("Game resumed.");

}

// =========================================================

//  Game over

// =========================================================

void endGame() {

game.active = false;

game.paused = false;

if (game.score1 > game.score2)      ledMode = LED_WIN_1;

else if (game.score2 > game.score1) ledMode = LED_WIN_2;

else                                ledMode = LED_DRAW;

// 3 victory beeps

for (int i = 0; i < 3; i++) {

buzzerOn();  delay(300);

buzzerOff(); delay(200);

}

Serial.printf("Game Over!  %s %d - %d %s\n",

game.name1.c_str(), game.score1, game.score2, game.name2.c_str());

}

// =========================================================

//  SETUP

// =========================================================

void setup() {

Serial.begin(115200);

pinMode(IR_SENSOR_1, INPUT_PULLUP);

pinMode(IR_SENSOR_2, INPUT_PULLUP);

pinMode(BUZZER_PIN,  OUTPUT);

buzzerOff();

// Start LED task on Core 0 (WiFi uses Core 1)

xTaskCreatePinnedToCore(

ledTask,     // function

"LEDTask",   // name

4096,        // stack size

NULL,        // params

1,           // priority

NULL,        // handle

0            // core 0

);

// Brief buzzer beep on boot

buzzerOn(); delay(200); buzzerOff();

// WiFi

Serial.printf("Connecting to %s", ssid);

WiFi.disconnect(true);

delay(500);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 20) {

delay(500);

Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.printf("\nConnected! IP: %s\n", WiFi.localIP().toString().c_str());

} else {

Serial.println("\nWiFi failed — check SSID/password.");

}

server.on("/api/start",  HTTP_POST,    handleStart);

server.on("/api/start",  HTTP_OPTIONS, handleOptions);

server.on("/api/stop",   HTTP_POST,    handleStop);

server.on("/api/stop",   HTTP_OPTIONS, handleOptions);

server.on("/api/state",  HTTP_GET,     handleState);

server.on("/api/state",  HTTP_OPTIONS, handleOptions);

server.begin();

Serial.println("Server ready.");

Serial.println(WiFi.localIP());

}

// =========================================================

//  LOOP — game logic only, LEDs handled by ledTask

// =========================================================

void loop() {

server.handleClient();

updateBuzzer();

if (!game.active) return;

unsigned long now = millis();

// Goal celebration pause

if (game.paused) {

if (now >= game.pauseEnd) resumeGame();

return;

}

// Countdown tick

if (now - game.lastTick >= 1000) {

game.lastTick = now;

if (game.timeLeft > 0) {

game.timeLeft--;

if (game.timeLeft == 0) {

endGame();

return;

}

}

}

// IR sensor polling with edge detection

bool s1 = (digitalRead(IR_SENSOR_1) == LOW);

bool s2 = (digitalRead(IR_SENSOR_2) == LOW);

if (!s1) sensor1Clear = true;

if (!s2) sensor2Clear = true;

if (s1 && sensor1Clear) triggerGoal(1);

if (s2 && sensor2Clear) triggerGoal(2);

}

Website code

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>Football Arena</title>

<link href="https://fonts.googleapis.com/css2?family=Bebas+Neue&family=Inter:wght@400;500;600&display=swap" rel="stylesheet">

<style>

*{box-sizing:border-box;margin:0;padding:0}

:root{--g:linear-gradient(135deg,#0f0f0f 0%,#1a1a2e 50%,#16213e 100%);--accent:#00ff88;--red:#ff3b5c;--gold:#ffd700;--card:rgba(255,255,255,0.04);--border:rgba(255,255,255,0.08)}

body{background:var(--g);min-height:100vh;font-family:'Inter',sans-serif;color:#fff;display:flex;flex-direction:column;align-items:center;padding:2rem 1rem}

h1{font-family:'Bebas Neue',sans-serif;font-size:3.2rem;letter-spacing:4px;background:linear-gradient(90deg,#00ff88,#00ccff);-webkit-background-clip:text;-webkit-text-fill-color:transparent;margin-bottom:.25rem;text-align:center}

.subtitle{font-size:.75rem;letter-spacing:3px;color:rgba(255,255,255,.35);text-transform:uppercase;margin-bottom:2rem;text-align:center}

#setup{width:100%;max-width:520px;display:flex;flex-direction:column;gap:1.25rem}

.field-group{display:flex;flex-direction:column;gap:.4rem}

label{font-size:.7rem;letter-spacing:2px;text-transform:uppercase;color:rgba(255,255,255,.45)}

input[type=text],input[type=number]{background:var(--card);border:1px solid var(--border);border-radius:8px;padding:.75rem 1rem;color:#fff;font-size:.95rem;font-family:inherit;outline:none;transition:border-color .2s;width:100%}

input:focus{border-color:var(--accent)}

.row{display:grid;grid-template-columns:1fr 1fr;gap:1rem}

.player-card{background:var(--card);border:1px solid var(--border);border-radius:12px;padding:1.25rem;display:flex;flex-direction:column;gap:.75rem}

.dot{width:10px;height:10px;border-radius:50%;margin-bottom:.25rem}

.dot-green{background:var(--accent);box-shadow:0 0 8px var(--accent)}

.dot-red{background:var(--red);box-shadow:0 0 8px var(--red)}

.btn-start{background:linear-gradient(135deg,var(--accent),#00ccff);color:#000;border:none;border-radius:10px;padding:1rem;font-size:1rem;font-weight:700;letter-spacing:2px;text-transform:uppercase;cursor:pointer;transition:transform .15s,opacity .2s;margin-top:.5rem}

.btn-start:hover{transform:scale(1.02)}

.btn-start:active{transform:scale(0.98)}

.btn-start:disabled{opacity:.4;cursor:not-allowed;transform:none}

.ip-row{display:flex;gap:.75rem;align-items:flex-end}

.ip-row input{flex:1}

.btn-connect{background:var(--card);border:1px solid var(--border);border-radius:8px;padding:.75rem 1.25rem;color:#fff;font-size:.8rem;font-family:inherit;cursor:pointer;white-space:nowrap;transition:border-color .2s}

.btn-connect:hover{border-color:var(--accent)}

.status-dot{width:8px;height:8px;border-radius:50%;background:#555;display:inline-block;margin-right:6px;vertical-align:middle;transition:background .3s}

.status-dot.ok{background:var(--accent);box-shadow:0 0 6px var(--accent)}

.status-dot.err{background:var(--red);box-shadow:0 0 6px var(--red)}

.status-label{font-size:.72rem;letter-spacing:1px;color:rgba(255,255,255,.4);display:flex;align-items:center}

#game{width:100%;max-width:700px;display:none;flex-direction:column;gap:1.5rem}

.timer-ring{text-align:center}

.timer-num{font-family:'Bebas Neue',sans-serif;font-size:5rem;letter-spacing:4px;line-height:1}

.timer-label{font-size:.65rem;letter-spacing:3px;color:rgba(255,255,255,.35);text-transform:uppercase;margin-top:.25rem}

.scoreboard{display:grid;grid-template-columns:1fr auto 1fr;gap:.5rem;align-items:center}

.player-panel{background:var(--card);border:1px solid var(--border);border-radius:16px;padding:1.5rem 1rem;display:flex;flex-direction:column;align-items:center;gap:.5rem;transition:all .3s;min-height:160px;justify-content:center}

.player-panel.scored{border-color:var(--red)!important;background:rgba(255,59,92,.08)!important}

.player-panel.winner{border-color:var(--gold);background:rgba(255,215,0,.06)}

.pname{font-size:.8rem;letter-spacing:2px;text-transform:uppercase;color:rgba(255,255,255,.5)}

.pscore{font-family:'Bebas Neue',sans-serif;font-size:4.5rem;line-height:1;transition:transform .2s}

.pscore.bump{transform:scale(1.25)}

.pled{width:100%;height:6px;border-radius:3px;background:#333;overflow:hidden}

.pled-fill{height:100%;border-radius:3px;transition:background .4s}

.vs{font-family:'Bebas Neue',sans-serif;font-size:1.8rem;color:rgba(255,255,255,.2);text-align:center}

.led-strips{display:grid;grid-template-columns:1fr 1fr;gap:.75rem}

.strip-wrap{border-radius:8px;overflow:hidden;height:12px;border:1px solid var(--border)}

.strip-inner{height:100%;width:100%;transition:background .4s}

.strip-label{font-size:.62rem;letter-spacing:2px;text-transform:uppercase;color:rgba(255,255,255,.3);margin-bottom:.35rem}

.goal-banner{text-align:center;padding:.75rem;border-radius:10px;font-family:'Bebas Neue',sans-serif;font-size:2rem;letter-spacing:4px;background:rgba(255,59,92,.12);border:1px solid rgba(255,59,92,.3);color:var(--red);display:none}

.goal-banner.show{display:block;animation:goalPulse 1s ease infinite}

@keyframes goalPulse{0%,100%{opacity:1}50%{opacity:.5}}

.gameover-panel{text-align:center;padding:2rem;background:var(--card);border:1px solid var(--border);border-radius:16px;display:none;flex-direction:column;gap:.75rem;align-items:center}

.gameover-panel.show{display:flex}

.go-title{font-family:'Bebas Neue',sans-serif;font-size:3rem;letter-spacing:4px;color:var(--gold)}

.go-winner{font-size:1.1rem;font-weight:600}

.btn-reset{background:var(--card);border:1px solid var(--border);border-radius:8px;padding:.75rem 2rem;color:#fff;font-size:.85rem;letter-spacing:2px;text-transform:uppercase;cursor:pointer;font-family:inherit;transition:border-color .2s;margin-top:.5rem}

.btn-reset:hover{border-color:var(--accent)}

.err-msg{color:var(--red);font-size:.75rem;letter-spacing:1px;text-align:center;min-height:1rem}

</style>

</head>

<body>

<h1>Football Arena</h1>

<p class="subtitle">Powered by ESP32</p>

<div id="setup">

<div class="field-group">

<label>ESP32 IP Address</label>

<div class="ip-row">

<input type="text" id="ipInput" placeholder="192.168.1.100">

<button class="btn-connect" onclick="testConnection()">Connect</button>

</div>

<div class="status-label"><span class="status-dot" id="connDot"></span><span id="connText">Not connected</span></div>

</div>

<div class="row">

<div class="player-card"><div class="dot dot-green"></div><div class="field-group"><label>Player 1 Name</label><input type="text" id="p1name" placeholder="Alice" maxlength="16"></div></div>

<div class="player-card"><div class="dot dot-red"></div><div class="field-group"><label>Player 2 Name</label><input type="text" id="p2name" placeholder="Bob" maxlength="16"></div></div>

</div>

<div class="field-group"><label>Game Duration (seconds)</label><input type="number" id="gametime" value="120" min="10" max="600"></div>

<p class="err-msg" id="errMsg"></p>

<button class="btn-start" id="btnStart" onclick="startGame()">Start Game</button>

</div>

<div id="game">

<div class="timer-ring"><div class="timer-num" id="timerNum">0:00</div><div class="timer-label">Time Remaining</div></div>

<div class="goal-banner" id="goalBanner">GOAL!</div>

<div class="scoreboard">

<div class="player-panel" id="panel1"><div class="pname" id="gname1">Player 1</div><div class="pscore" id="gscore1">0</div><div class="pled"><div class="pled-fill" id="led1bar" style="width:6px;background:var(--accent)"></div></div></div>

<div class="vs">VS</div>

<div class="player-panel" id="panel2"><div class="pname" id="gname2">Player 2</div><div class="pscore" id="gscore2">0</div><div class="pled"><div class="pled-fill" id="led2bar" style="width:6px;background:var(--red)"></div></div></div>

</div>

<div>

<div class="strip-label">LED Strips</div>

<div class="led-strips">

<div><div class="strip-label" id="slabel1">Player 1</div><div class="strip-wrap"><div class="strip-inner" id="strip1vis"></div></div></div>

<div><div class="strip-label" id="slabel2">Player 2</div><div class="strip-wrap"><div class="strip-inner" id="strip2vis"></div></div></div>

</div>

</div>

<div class="gameover-panel" id="gameoverPanel">

<div class="go-title">Full Time!</div>

<div class="go-winner" id="goWinner"></div>

<button class="btn-reset" onclick="resetGame()">New Game</button>

</div>

</div>

<script>

let esp32IP='',pollTimer=null,lastScore1=0,lastScore2=0,inRainbow=false,rainbowInterval=null,gameRunning=false,rainbowHue=0;

function getBase(){return`http://${esp32IP}`}

async function testConnection(){

const ip=document.getElementById('ipInput').value.trim();

if(!ip){showErr('Enter IP address');return}

esp32IP=ip;

const dot=document.getElementById('connDot'),txt=document.getElementById('connText');

txt.textContent='Connecting...';dot.className='status-dot';

try{const r=await fetch(`http://${ip}/api/state`,{signal:AbortSignal.timeout(3000)});

if(r.ok){dot.className='status-dot ok';txt.textContent=`Connected to ${ip}`}else throw new Error()

}catch{dot.className='status-dot err';txt.textContent='Connection failed - check IP';esp32IP=''}

}

function showErr(msg){document.getElementById('errMsg').textContent=msg;setTimeout(()=>document.getElementById('errMsg').textContent='',3000)}

async function startGame(){

const ip=document.getElementById('ipInput').value.trim();

if(!ip){showErr('Connect to ESP32 first');return}

esp32IP=ip;

const n1=document.getElementById('p1name').value.trim()||'Player 1';

const n2=document.getElementById('p2name').value.trim()||'Player 2';

const t=parseInt(document.getElementById('gametime').value)||120;

document.getElementById('btnStart').disabled=true;

try{

const r=await fetch(`${getBase()}/api/start`,{method:'POST',headers:{'Content-Type':'application/json'},body:JSON.stringify({name1:n1,name2:n2,time:t})});

if(!r.ok)throw new Error();

lastScore1=0;lastScore2=0;

['gname1','gname2'].forEach((id,i)=>document.getElementById(id).textContent=[n1,n2][i]);

['slabel1','slabel2'].forEach((id,i)=>document.getElementById(id).textContent=[n1,n2][i]);

['gscore1','gscore2'].forEach(id=>document.getElementById(id).textContent='0');

document.getElementById('gameoverPanel').classList.remove('show');

document.getElementById('goalBanner').classList.remove('show');

document.getElementById('setup').style.display='none';

document.getElementById('game').style.display='flex';

gameRunning=true;

startRainbow();

setTimeout(()=>{stopRainbow();setStripsWhite()},3000);

startPolling();

}catch{showErr('Failed to start - check connection');document.getElementById('btnStart').disabled=false}

}

function startPolling(){if(pollTimer)clearInterval(pollTimer);pollTimer=setInterval(poll,500)}

async function poll(){

if(!gameRunning)return;

try{const r=await fetch(`${getBase()}/api/state`,{signal:AbortSignal.timeout(2000)});updateUI(await r.json())}catch{}

}

function updateUI(d){

const m=Math.floor(d.timeLeft/60),s=d.timeLeft%60;

document.getElementById('timerNum').textContent=`${m}:${s.toString().padStart(2,'0')}`;

if(d.score1!==lastScore1){document.getElementById('gscore1').textContent=d.score1;bumpAnim('gscore1');lastScore1=d.score1}

if(d.score2!==lastScore2){document.getElementById('gscore2').textContent=d.score2;bumpAnim('gscore2');lastScore2=d.score2}

const total=Math.max(d.score1+d.score2,1);

document.getElementById('led1bar').style.width=`${Math.max(8,(d.score1/total)*100)}%`;

document.getElementById('led2bar').style.width=`${Math.max(8,(d.score2/total)*100)}%`;

const p1=document.getElementById('panel1'),p2=document.getElementById('panel2'),banner=document.getElementById('goalBanner');

if(d.paused&&d.winner>0){

banner.textContent=`GOAL! ${d.winner==1?d.name1:d.name2} scored!`;

banner.classList.add('show');

if(d.winner==1){p1.classList.add('scored');p2.classList.remove('scored');setStrip1('red');setStrip2('off')}

else{p2.classList.add('scored');p1.classList.remove('scored');setStrip2('red');setStrip1('off')}

}else if(!d.paused&&d.active){

banner.classList.remove('show');p1.classList.remove('scored');p2.classList.remove('scored');

if(!inRainbow){setStrip1('white');setStrip2('white')}

}

if(!d.active&&gameRunning){

gameRunning=false;stopRainbow();clearInterval(pollTimer);

banner.classList.remove('show');p1.classList.remove('scored');p2.classList.remove('scored');

const go=document.getElementById('gameoverPanel'),gw=document.getElementById('goWinner');

if(d.score1>d.score2){gw.textContent=`${d.name1} wins! ${d.score1} - ${d.score2}`;setStrip1('green');setStrip2('red');p1.classList.add('winner')}

else if(d.score2>d.score1){gw.textContent=`${d.name2} wins! ${d.score2} - ${d.score1}`;setStrip2('green');setStrip1('red');p2.classList.add('winner')}

else{gw.textContent=`It's a draw! ${d.score1} - ${d.score2}`;setStrip1('yellow');setStrip2('yellow')}

go.classList.add('show');

}

}

function bumpAnim(id){const el=document.getElementById(id);el.classList.add('bump');setTimeout(()=>el.classList.remove('bump'),300)}

function startRainbow(){

inRainbow=true;if(rainbowInterval)clearInterval(rainbowInterval);

rainbowInterval=setInterval(()=>{

rainbowHue=(rainbowHue+3)%360;

document.getElementById('strip1vis').style.background=`hsl(${rainbowHue},100%,50%)`;

document.getElementById('strip2vis').style.background=`hsl(${(rainbowHue+120)%360},100%,50%)`;

},40);

}

function stopRainbow(){inRainbow=false;if(rainbowInterval){clearInterval(rainbowInterval);rainbowInterval=null}}

function setStripsWhite(){document.getElementById('strip1vis').style.background='#ffffff';document.getElementById('strip2vis').style.background='#ffffff'}

function setStrip1(c){document.getElementById('strip1vis').style.background=colorMap(c)}

function setStrip2(c){document.getElementById('strip2vis').style.background=colorMap(c)}

function colorMap(c){return{white:'#ffffff',red:'#ff3b5c',off:'#111111',green:'#00ff88',yellow:'#ffd700'}[c]||'#111'}

async function resetGame(){

try{await fetch(`${getBase()}/api/stop`,{method:'POST'})}catch{}

gameRunning=false;stopRainbow();if(pollTimer)clearInterval(pollTimer);

['panel1','panel2'].forEach(id=>document.getElementById(id).classList.remove('winner','scored'));

document.getElementById('strip1vis').style.background='#111';

document.getElementById('strip2vis').style.background='#111';

document.getElementById('game').style.display='none';

document.getElementById('setup').style.display='flex';

document.getElementById('btnStart').disabled=false;

}

</script>

</body>

</html>